����������������������������������������������������������������������������ͻ

�         HWiNFO � Copyright (c)1995-2010 Martin Mal�k - REALiX(tm)          �

� �������������������������������������������������������������������������� �

�                # Version 5.5.0; Last Update: Jun-08-2010 #                 �

����������������������������������������������������������������������������ͼ

������������������������������������������Ŀ

�  USE THIS PROGRAM ONLY AT YOUR OWN RISK  �

��������������������������������������������

CONTENTS

1.  Intro

2.  Updates and download

3.  Requirements

4.  Utilities distributed with HWiNFO

5.  Registration

6.  Creating a logfile

7.  Limitations

8.  Running HWiNFO under V86 Memory Managers and Windows

9.  Resetting the CPU

10.  HWiNFO Crash

13.  Hardware monitor chip support

14.  IDE Disk Transfer Modes

15.  Miscellaneous notes

16.  Some abbreviations

18.  Disclaimer and Terms of License

## 1. Intro ##################################################################

HWiNFO is a *POWERFUL* hardware detection program. It is simple, quick and

on most systems very efficient. It "tries" to give only important information

about all the hardware devices in your computer so that you need not look

inside the PC to see what's there.

## 2. Updates and download ###################################################

The newest version is always (immediately) available at:

HWiNFO Homepage EN - http://www.hwinfo.com

HWiNFO Homepage SK - http://www.hwinfo.sk

SAC ftp            - ftp.elf.stuba.sk/pub/pc/utildiag

SAC WWW            - www.sac.sk

## 3. Requirements ###########################################################

- 80386 or better CPU

- cca. 500 KBytes of free Conventional system memory

- cca. 2.0 MBytes of free XMS memory (XMS manager recommended)

## 4. Creating a logfile #####################################################

There are three possible methods how to create a logfile:

1. Simple log

After pressing the <F2> key, HWiNFO adds the results from current screen

to a logfile.

2. Report file ("-r" switch) [REGISTERED VERSION ONLY]

HWiNFO creates AUTOMATICALLY a COMPLETE logfile at startup.

This logfile has the same format as the one created with <F2> key, but

there's no need to navigate HWiNFO through all the screens.

3. Short report file ("-s" switch) [REGISTERED VERSION ONLY]

HWiNFO creates AUTOMATICALLY a logfile, which contains only the

MOST IMPORTANT items. This format is suitable for further automatical

processing (or printing) of logfiles (e.g. if you would like to create

an inventory of more machines). Here's a sample of the format:

CPU=Intel Pentium III Xeon, 550.0 MHz

Bus=ISA + PCI + A.G.P.

Chipset=Intel 82440GX AGPset

Memory=128 MBytes

...

VideoChipset=nVIDIA GeForce 256 DDR

VideoMemory=32768 KBytes of SGRAM

3Daccel=3Dfx Voodoo 2

...

If you use a capital letter (either "-R" or "-S") the benchmark will not

be performed.

The "-v" switch enables Verbose Mode, where all screen outputs are supressed.

The default log filename is "HWiNFO.LOG". You can change its name if you

enter it as a paramater to HWiNFO (e.g. "HWiNFO.EXE -S -v HOME.LOG").

Samples of logfiles can be found at HWiNFO homepage.

## 5. Limitations ############################################################

The Mainboard Chipset identification works only on newer (1991 and above)

AMI and newer AWARD BIOSes. HWiNFO is also able to detect some chipsets, not

depending on the type of BIOS (see HWiNFO.LST).

PCMCIA detection requires Socket Services or Card Services to be installed.

If you want more info about sockets, install Card Services.

The VESA Local Bus can be detected only if a VLB card is present.

## 6. Running HWiNFO under V86 Memory Managers and Windows ###################

In order to achieve best results, HWiNFO accesses directly the hardware.

Some of the routines (privileged) require to be run in Real mode or at

privilege level 0 (PL0, most privileged), but in V86 mode are all

applications run at PL3. This means, if such a routine is run, the CPU

raises an exception, which is handled by the V86 manager. Note, that

V86 mode is the only way to run a DOS application (Real mode) under

MS Windows (Protected mode). EMM (Expanded Memory Manager) also use the

V86 mode to provide paging.

But HWiNFO v3.00+ is able to run the privileged routines in V86 mode under

some circumstances. There is a special call (called WINDOWS INIT BROADCAST),

which can be used to force an V86 manager to switch to Real mode (of course,

the V86 manager must support it). This call was originally used by

MS Windows while starting, while an Expanded Memory Manager was running

(this is needed, because EMM is running in Protected mode and Windows too).

Such known EMM managers are:

- Microsoft EMM386 Expanded Memory Manager

- Quarterdeck QEMM386 Expanded Memory Manager

- Qualitas 386MAX Expanded Memory Manager

- Nu-Mega Soft-ICE Debugger

...

So HWiNFO uses this call to force the manager to switch back to Real mode.

HWiNFO works best under EMM386 (I haven't found any problems).

But there are some disadvantages while runnig in V86 mode:

- Some V86 managers are able to relocate BIOSes, so HWiNFO won't be able

to identify BIOS type and some video cards.

- A crash while detecting the CPU type could happen on some older

CPUs requiring CPU RESET to identify them properly.

The other case is, if a manager doesn't support the call. This happens if

you are running under Microsoft Windows95 OSR2 or Windows98 DOS mode.

If these managers receive the Windows INIT BCAST, they display a message

about running an incompatible application and stop. So the only way is

to skip the privileged routines.

Pre v4.4.1 of HWiNFO refused to run under this circumstances, but

later versions run with the following restrictions (skip privileged routs):

- CPU frequency can't be measured using TSC

- CPU RESET can't be performed

(this is needed only for older CPUs or to receive the Processor Number

while this feature has been disabled)

- TI486 Potomac CPUs can't be identified

- L2 cache ECC presence can't be detected (P-II, Xeon)

- CPU Update Revision can't be reported (PPro and above)

If you try to run HWiNFO directly from Microsoft Windows (DOS box), it will

refuse to continue. This is because Windows controls also access to some

ports and doesn't allow a direct access from a DOS application. This

means, that if HWiNFO wouldn't refuse to run, but continue, it would be

unable to return info about IDE/ATAPI drives, COM/LPT ports, BIOS scan,

PnP resources, Benchmarks wouldn't be accurate...

Windows NT VDM (Virtual DOS Machine) is even more strict.

## 7. Resetting the CPU ######################################################

The CPU ID is available only on 386+ CPUs and HWiNFO uses three ways to

obtain this ID:

Method 1 - Resets the CPU while A20 is Enabled.

This method works on all systems, but the Reset handler gets

control after BIOS. And some BIOSes (some newer AMI/Award BIOSes)

destroy CPU ID (EDX) and therefore this ID could be invalid.

This method is default <Enabled> in Setup section.

Method 2 - Resets the CPU while A20 is Disabled.

This method works only on some machines, which do have a

Fully-Terminated bus. It's the best way to obtain CPU ID, because

it gets control immediately after Reset (the BIOS can't destroy

EDX). I've found some problems with this method while running

with Norton cache.

This method is default <Disabled> in Setup section.

Method 3 - Try to use this method if your BIOS destroys CPU ID. But it works

only on systems, which do not have Main BIOS (area at 0F000:0)

Write-Protected, because it modifies it for a bit, and then

restores back. This method is on some machines the only way

to obtain CPU ID.

This method is default <Enabled> in Setup section.

HWiNFO tries to determine CPU ID this way, via: 1. CPUID instruction

2. BIOS (INT 15h)

3. Method 2 (if Enabled)

4. Method 3

5. Method 1

If you have an UMC U5SX/SD/SLV/486DX2/SX2, Enhanced Am486DX2/DX4/DX4-WB,

AMD-K5 and above, Intel 486DX/SX/DX2 SL-Enhanced, IntelDX4, IntelSX2,

IntelDX2 W-B Enh. or Intel Pentium(R) and above, there is no need to Reset

the CPU (these CPU support CPUID instruction). Most CPUs today support

CPUID instruction.

Note: NexGen Nx586 does NOT support the CPUID instruction.

Cyrix 5x86 and above support the CPUID instruction only if CPUIDEN bit

in CCR4 register is set. HWiNFO 3.0.7+ sets this bit automatically.

Disabling "CPU ID detection via Method 1, 2 and 3" reduces your CPU

detection ability. And if there's no other way to obtain this ID

(via BIOS or CPUID instruction), it won't identify 386SL, i386CX/EX,

486DX2, IBM 486... and some other CPUs. Do this only if your system

hangs while attempting CPU detection.

Some BIOSes simply OVERWRITE the CPU ID (EDX) :-< ! In this case an

invalid CPU ID is displayed and the CPU is probably incorrectly identified.

If this problem occurs, try to use Method 3 (but your Main BIOS must be

Write-Enabled !). If it doesn't work, try to use Method 2 (but you must

have a Fully-Terminated bus !). And if HWiNFO still displays, that Method 1

was used to obtain CPU ID, not Method 2 nor 3 can obtain it.

## 8. HWiNFO Crash ###########################################################

- Stacker Compression Card and 3Com EtherLink III Adapter detection

routines can cause crashes with some Network cards. You can disable

them in the Configuration section (Stacker check is default disabled;

3Com EtherLink III detection is enabled).

- If HWiNFO crashes while starting or while the "Flushing Disk Caches..."

message is displayed, disable the "FlushDiskCache" option in HWiNFO.INI

or HWiNFO|Setup.

## 9. Miscellaneous notes ####################################################

On some old motherboards, random numbers are obtained from unused I/O ports

and this may cause invalid identification of some hardware (e.g. WSS).

I don't know how to solve this problem.

Intel Pentium(R) and above CPUs do not report, that the "APIC on-chip"

feature is present. Most of them (P54C yes, but P54CQS and P54LM don't)

have an on-chip APIC, but don't report it, if it isn't supported by

external hardware.

If you have an Intel 420LX/420NX/430FX/430HX/430VX/430MX/430TX,

SiS 85C501/5120/5511/5571/5581/5596/5597/5591, AMD-640 chipset or

VIA Apollo VP-1/VPX/VP-2/VP3/MVP3, HWiNFO is able to detect the type of

External Cache and its size direct from the PCI chipset registers.

## 10. Some abbreviations ####################################################

FPM           = Fast-Page Mode (old standard DRAM type)

EDO           = Extended Data-Out

SDRAM         = Synchronous DRAM

DDR SDRAM II  = Double Data Rate SDRAM II

BDDR SDRAM II = Bi-directionally strobed DDR SDRAM II

BEDO          = Burst EDO

EC            = Error Check only

ECC           = Error Check and Correction

DRAM          = Dynamic RAM

VRAM          = Video RAM

WRAM          = Window RAM

SDRAM         = Synchronous DRAM

SGRAM         = Synchronous Graphics RAM

RDRAM         = Rambus DRAM

MDRAM         = Multibank DRAM

VRT           = Voltage Reduction Technology

APIC          = Advanced Programmable Interrupt Controller (required for DP)

DP            = Dual Processing

FRC           = Functional Redundancy Checking (requires DP)

MMX           = Multimedia Extensions

(57 new instructions, SIMD architecture, 64-bit regs)

MMX-2         = Some enhancements to MMX (Intel)

3DNow!        = Additional floating point MMX instructions (AMD, Cyrix)

ISSE / KNI    = Internet Streaming SIMD Extensions (Katmai New Instructions)

(70 SIMD floating-point instructions, 128-bit registers)

S.E.C.C.      = Single Edge Contact Cartridge (SECC1, SECC2)

PPGA          = Plastic PGA (Pin Grid Array)

FCPGA         = Flip-Chip PGA

MPS           = Multi-Processor System

P.I.ROM       = Processor Information ROM

TLB           = Translation Lookaside Buffer

(small cache for page translation)

PnP           = Plug-and-Play

ESCD          = Extended System Configuration Data

DMI           = Desktop Management Interface

ACPI          = Advanced Configuration and Power Interface

APM           = Advanced Power Management

VESA/VBE      = Video Electronics Standards Association /

Video BIOS Extensions

DDC           = Display Data Channel

(DDC1: Monitor->PC, DDC2B: Monitor<->PC)

LFM           = Low-Frequency Mode (usually the minimum CPU operating clock)

SLFM          = Super-LFM Mode (Dynamic FSB switching mode)

HFM           = High-Frequency Mode (Maximum Non-Turbo mode, marketed)

IDA           = Intel Dynamic Acceleration (Turbo mode)

## 11. Disclaimer ############################################################

����������������������������������������������������������������������������ͻ

�  All product names are copyright and registered trademarks/tradenames of   �

�                        their respective owners.                            �

����������������������������������������������������������������������������ͼ

HWiNFO IS COPYRIGHTED MATERIAL AND PROTECTED BY APPLICABLE

COPYRIGHT LAWS AND INTERNATIONAL TREATY PROVISIONS.

THE PROGRAM AND ITS DOCUMENTATION ARE COPYRIGHTED BY THE ORIGINAL

PROGRAMMER/AUTHOR (COPYRIGHT HOLDER). ALL RIGHTS ARE RESERVED CONCERNING

FUTURE CHANGES OF THE NAME AND CONTENTS OF THIS DISTRIBUTION PACKAGE. NO

CHANGES MUST BE MADE FOR ANY REASONS AT ANY TIME BY ANYBODY IN ANY ASPECT,

INCLUDING, BUT NOT LIMITED TO, MODIFICATION OF THE COPYRIGHT NOTICE OR

CHANGES OF THE COMPILED BINARIES HEREBY ALTERING THE ORIGINALLY INTENDED

PROGRAM EXECUTION FLOW.

REALiX SPECIFICALLY DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, AND

ALL LIABILITY, INCLUDING CONSEQUENTIAL AND OTHER INDIRECT DAMAGES, FOR

THE USE OF THIS SOFTWARE, INCLUDING LIABILITY FOR INFRINGEMENT OF ANY

PROPRIETARY RIGHTS. REALiX DOES NOT ASSUME ANY RESPONSIBILITY FOR ANY

ERRORS WHICH MAY APPEAR IN THIS CODE NOR ANY RESPONSIBILITY TO UPDATE IT.

THE PERSON USING THE SOFTWARE BEARS ALL RISKS AS TO THE QUALITY AND

PERFORMANCE OF THIS SOFTWARE.

## END #######################################################################